CECS 447 Project 3: A Bluetooth Controlled Robot Car

By: Min He

Total: 20 points

Preparation

You will need one LaunchPad, a robot car driven by two DC motors, one L298N Motor Drive Controller, at least 6 AA batteries to power the car and the Launchpad, and one HC-05 Bluetooth module. You will use a serial terminal application, such as Tera term, Putty, Realterm, etc., which can perform serial port communication on the PC. A smartphone/Laptop is required to send commands to the Robot car through a Bluetooth terminal application.

# Reference/Starter Projects: 1. BlueToothSeup, 2. BlueTooth Controlled LEDs, 3. Romi_Car_Test

# Project Description:

In this project, you will build a robot car and use a cell phone/PC Bluetooth terminal to control the movement of the car. You will also use the onboard LEDs to indicate the status of the car: moving forward, moving backward, stop, etc. The goal of this project is to learn how to interface an HC-05 Bluetooth module through UART, including setting up an HC-05 Bluetooth module and  using the module to receive information from a cell phone/PC Bluetooth terminal to control a robot car.

# Project Requirements:

# You will build two projects: the first one is used to setup your HC-05 Bluetooth module, and the second program will use this Bluetooth module to receive control code from a cell phone/PC and control a robot car movement based on the control code received. Your DC motors should be controlled by hardware PWM signals.

# Part 1(5 points): Setup Bluetooth Module

Use UART1 to communicate with HC-05 Bluetooth module. Set Bluetooth Module to Command Mode: connect the HC-05 Bluetooth module to your Microcontroller with the following connections:

| HC-05 Bluetooth Module | Launchpad |
| --- | --- |
| State: leave unconnected |  |
| RX | UART1 TX |
| TX | UART1 RX |
| GND | GND |
| VCC | 5v (VBUS) |
| EN/KEY: | 3.3v or 5v |

Notice that we need to connect EN/KEY pin to 3.3-5V before powering chip to get into Command Mode. If there is a button on your board connect EN/KEY to 3.3-5V press and hold button before applying power to get into Command Mode. There are two firmware versions: 2.0 and 4.0. The example setup code works for 2.0. Use the example setup code to test your BlueTooth module, then write a program with user interface to change your HC-05 Bluetooth module setting to the following configurations:

  - Bluetooth name to your name

  - UART Configuration: 57600, 8 Data bits, 1 stop, odd parity

  - Passcode: you define this, should not be 1234

  - Set to Slave mode

>>> Welcome to Serial Terminal. <<<

>>> This is the setup program for HC-05 Bluetooth module. <<<

>>> You are at ‘AT’ Command Mode. <<<

>>> Type 'AT' and followed with a command. <<<

>>> Example: AT+NAME=Your Name <<<

AT+NAME=Your Name

OK AT+NAME?

+NAME Your Name

OK

AT+UART=57600,0,1OK

AT+UART?

+UART=57600,0,1 OK AT+PSWD=4321 OK

AT+PSWD?

+PSWD: 4321 OK AT+ROLE=0 OK

AT+ROLE?

+ROLE : 0 OK

The default baud rate of the HC-05 Bluetooth module is 38400 for command mode and 9600 for data communication mode. Your setup program should run in the following way: After putting Bluetooth in AT command mode and send the first command: AT, your program will stay in super loop waiting for user input for AT commands. User can enter AT command from Tera term or any other serial communication software. Record your serial terminal input and output data while you are configuring the device in command mode and put it in the “Operation” section in your project report. Below is an example of the serial terminal display: Green text are program prompt to user; purple text are user input; blue text are responses received from the Bluetooth module.

Part 2: A Bluetooth Controlled Robot Car 
(10 points: 5 points each modes):

Your BlueTooth controlled robot car will work on the following two modes:

- Mode 1: Demo

- Mode 2: Free Driving

Set Bluetooth Module to Data Mode (disconnect the EN/KEY pin) and write a program for your robot car to serially communicate with a smartphone/PC Bluetooth APP through the HC-05 Bluetooth module. You can either download or write your own Bluetooth terminal application on your smartphone, and pair your phone with your Bluetooth module. The robot car will be in Mode 1 when powering up. Use onboard switch 1 to toggle between the two modes. When in Mode 1, turn on green LED, when in Mode 2, turn on blue

M0

LED.

HC-05 Bluetooth

TM4C123

Motor Driver

M1

2.4GHz Wireless

Bluetooth Terminal

Mode 1: Demo

Send the following characters as commands one at a time from your BlueTooth serial terminal to your microcontroller to do the following demonstrations:

    - ‘8’: draw one figure 8 and then stop.

    - ‘C’: draw one circle and then stop.

    - ‘S’: draw one square and then stop.

    - ‘Z’: draw a zigzag with 4 segments and then stop.

Mode 2: Free Driving

Send the following characters as commands one at a time from your BlueTooth serial terminal to your microcontroller to   control your robot car movements:

    - ‘F’: Forward

    - ‘B’: Reverse

    - ‘L’: Left Wide Turn

    - ‘R’: Right WideTurn

    - ‘S’: Stop

    - ‘U’: Speed Up

    - ‘D’: Slow down

# Deliverables (5 points):

- Demonstrate both parts to your instructor.

- Submit a video or a link to your video that records all the required behavior of your embedded system with both Bluetooth terminal APP and your robot car shown in the video. If a link is chosen, please add the link to the operation section of your project report.

- Submit a project report: follow the project report template for report format. The following information are specific to this project and are required to add to your report:

  - Operations: Steps to setup the HC-05 Bluetooth module, including hardware connection, steps to pair your Bluetooth module at your cell phone/laptop, and steps to send command from your Bluetooth APP.

  - Hardware design: include both setup hardware connection picture and schematic and data communication mode hardware connection picture and schematic.

- Submit all source code: including setup code and Bluetooth controlled robot car code.